THE UNHEADED KINGDOM

ASSEMBLY LANGUAGE TUTORIAL SERIES

Part 2 of 4

ICMP Protocol
Structure

Message Types, Codes, Headers, Ping/Traceroute Mechanics, and CLI Tools

For Infrastructure Engineers

February 2026

bellis.tech

# The ICMP Header

Every ICMP message has a simple 8-byte header followed by variable data:

0                   1                   2                   3

0 1 2 3 4 5 6 7 8 9 0 1 2 3 4 5 6 7 8 9 0 1 2 3 4 5 6 7 8 9 0 1

+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+

|     Type      |     Code      |          Checksum             |

+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+

|                     Type-specific data                        |

+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+

|                     Original packet (for errors)              |

|              (IP header + first 8 bytes of payload)           |

+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+

| Field | Size | Purpose |
| --- | --- | --- |
| Type | 1 byte | Category of message (Echo Request=8, Echo Reply=0, etc.) |
| Code | 1 byte | Sub-type within the category (e.g., Port Unreachable=3 within Destination Unreachable) |
| Checksum | 2 bytes | Error detection over the entire ICMP message |
| Type-specific | 4 bytes | Varies: identifier+sequence for ping, MTU for Packet Too Big, unused/pointer for errors |

For error messages, ICMP includes the original packet’s IP header plus the first 8 bytes of its payload. This lets the sender identify WHICH packet caused the error.

# ICMPv4 Message Types

## Error Messages (Type 0–127 — but specific types used)

| Type | Name | Common Codes |
| --- | --- | --- |
| 3 | Destination Unreachable | 0=Net, 1=Host, 2=Protocol, 3=Port, 4=Frag needed (DF set), 13=Admin prohibited |
| 5 | Redirect | 0=Network, 1=Host, 2=ToS+Network, 3=ToS+Host |
| 11 | Time Exceeded | 0=TTL expired in transit (traceroute!), 1=Fragment reassembly timeout |
| 12 | Parameter Problem | 0=Pointer indicates error field in header |

## Informational Messages

| Type | Name | Purpose |
| --- | --- | --- |
| 0 | Echo Reply | Response to ping |
| 8 | Echo Request | Ping — "Are you there?" |
| 13/14 | Timestamp Request/Reply | Measure one-way delay (rarely used) |
| 9/10 | Router Advertisement/Solicitation | Router discovery (mostly replaced by DHCP) |

# ICMPv6 Message Types

ICMPv6 is a much bigger protocol than ICMPv4. It handles errors AND replaces ARP, DHCP discovery, and multicast management:

## Error Messages (Type 1–127)

| Type | Name | Key Codes |
| --- | --- | --- |
| 1 | Destination Unreachable | 0=No route, 1=Admin prohibited, 3=Address unreachable, 4=Port unreachable |
| 2 | Packet Too Big | Code 0 only; includes MTU of the bottleneck link |
| 3 | Time Exceeded | 0=Hop limit exceeded, 1=Fragment reassembly timeout |
| 4 | Parameter Problem | 0=Erroneous header field, 1=Unknown next header, 2=Unknown option |

## Informational Messages (Type 128–255)

| Type | Name | Purpose |
| --- | --- | --- |
| 128/129 | Echo Request/Reply | ping6 |
| 130–132 | MLD (Multicast Listener Discovery) | Replaces IGMP for IPv6 multicast group management |
| 133 | Router Solicitation (RS) | Host asks: "Any routers here?" |
| 134 | Router Advertisement (RA) | Router announces prefix, flags, lifetime, DNS |
| 135 | Neighbor Solicitation (NS) | "Who has this IPv6 address?" (replaces ARP) |
| 136 | Neighbor Advertisement (NA) | "I have it! Here’s my MAC address." |
| 137 | Redirect | "Use this better next hop" |

# How Ping Works (Detail)

Echo Request (Type 8/128):

+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+

| Type=8 (v4)  |    Code=0     |          Checksum             |

|   or 128(v6) |               |                               |

+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+

|         Identifier            |       Sequence Number         |

+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+

|                     Payload (variable)                        |

|            (timestamp + padding, typically 56 bytes)          |

+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+

Echo Reply (Type 0/129):

Same format, just Type changes. All other fields echoed back.

| Field | Purpose |
| --- | --- |
| Identifier | Matches requests to replies (usually PID of ping process) |
| Sequence Number | Increments with each ping; detects loss and reordering |
| Payload | Timestamp (for RTT calculation) + padding bytes |

# How Traceroute Works (Detail)

TTL=1:  Packet -> Router1 -> TTL=0! -> ICMP Time Exceeded back to you

"Router1 is at 10.0.0.1, took 2.1ms"

TTL=2:  Packet -> Router1 -> Router2 -> TTL=0! -> ICMP Time Exceeded

"Router2 is at 10.1.0.1, took 8.4ms"

TTL=3:  Packet -> Router1 -> Router2 -> Router3 -> TTL=0!

"Router3 is at 10.2.0.1, took 12.1ms"

TTL=N:  Packet reaches destination -> ICMP Echo Reply (or Port Unreachable)

"Destination reached in N hops!"

## Traceroute Variants

| Tool | Method | Advantage |
| --- | --- | --- |
| traceroute (Unix) | UDP to high ports (33434+) | Default; port unreachable at destination signals arrival |
| traceroute -I | ICMP Echo Request | More reliable through firewalls that allow ping |
| traceroute -T | TCP SYN to port 80/443 | Best firewall traversal; looks like web traffic |
| traceroute6 / traceroute -6 | ICMPv6 over IPv6 | IPv6 path tracing |
| mtr | Continuous traceroute + ping combined | Real-time; shows loss and jitter per hop |
| paris-traceroute | Fixed flow ID per probe | Avoids ECMP path variation between probes |

# CLI Commands

## Ping

# Basic ping

ping 8.8.8.8                    # IPv4

ping6 2001:4860:4860::8888      # IPv6

ping -6 google.com              # Force IPv6

# Control count and interval

ping -c 10 -i 0.5 8.8.8.8      # 10 pings, 0.5s apart

# Set packet size

ping -s 1472 -M do 8.8.8.8     # 1472 bytes + 28 header = 1500 MTU test

# -M do = Don't Fragment

# Flood ping (requires root)

sudo ping -f -c 1000 192.168.1.1

## Traceroute

traceroute google.com           # UDP (default)

traceroute -I google.com        # ICMP

traceroute -T -p 443 google.com # TCP SYN

traceroute6 google.com          # IPv6

mtr google.com                  # Continuous (best tool!)

## Packet Capture

# Capture all ICMP traffic

sudo tcpdump -i eth0 icmp        # ICMPv4

sudo tcpdump -i eth0 icmp6       # ICMPv6

# Capture only ping

sudo tcpdump -i eth0 'icmp[icmptype] == 8 or icmp[icmptype] == 0'

# Capture Destination Unreachable

sudo tcpdump -i eth0 'icmp[icmptype] == 3'

“The Pattern echoes in every error message.” — The Whispering Void