THE UNHEADED KINGDOM

ASSEMBLY LANGUAGE TUTORIAL SERIES

Part 3 of 4

ICMP Protocol
Internals

Path MTU Discovery, ICMP Rate Limiting, NDP Deep Dive,
Firewall Rules, Security Hardening, and Monitoring

For Infrastructure Engineers

February 2026

bellis.tech

# Path MTU Discovery (PMTUD)

In IPv4, routers can fragment oversized packets. In IPv6, they cannot — only the sender fragments. PMTUD discovers the maximum packet size that will traverse the entire path without fragmentation:

## How PMTUD Works

1. Sender transmits packet with DF (Don't Fragment) bit set (IPv4)

or just sends normally (IPv6 never fragments at routers)

2. If packet > link MTU at any router:

IPv4: Router sends ICMP Type 3, Code 4 (Frag Needed) with Next-Hop MTU

IPv6: Router sends ICMPv6 Type 2 (Packet Too Big) with MTU field

3. Sender reduces packet size to reported MTU

4. Repeat until packets traverse entire path

5. Cache the discovered PMTU per destination (Linux: ip route show cache)

## PMTUD Packet Too Big Message

ICMPv6 Packet Too Big (Type 2):

+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+

|     Type=2    |     Code=0    |          Checksum             |

+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+

|                         MTU (4 bytes)                         |

+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+

|              Original IPv6 Header + Payload                   |

|         (as much as will fit without exceeding 1280)          |

+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+

## Common MTU Values

| Link Type | MTU | With Encapsulation |
| --- | --- | --- |
| Ethernet | 1500 | Standard |
| Jumbo frames | 9000 | Datacenter only; must be configured end-to-end |
| PPPoE (DSL/fiber) | 1492 | 8-byte PPPoE header |
| VXLAN over IPv4 | 1450 | 1500 - 50 (VXLAN+UDP+outer IP) |
| VXLAN over IPv6 | 1430 | 1500 - 70 (VXLAN+UDP+outer IPv6) |
| WireGuard over IPv4 | 1420 | 1500 - 80 (WG+UDP+outer IP) |
| IPv6 minimum | 1280 | Minimum MTU any IPv6 link must support |

## PMTUD Black Holes

If a firewall drops ICMPv6 Packet Too Big messages, the sender never learns the true MTU. Large packets are silently dropped. TCP connections stall because segments are lost without feedback. This is a PMTUD black hole.

- Diagnosis: ping -s 1472 -M do target works but large TCP transfers stall.

- Fix: NEVER block ICMPv6 Type 2 (Packet Too Big). Ever.

- Workaround: TCP MSS clamping at the firewall/router: iptables -A FORWARD -p tcp --tcp-flags SYN,RST SYN -j TCPMSS --clamp-mss-to-pmtu

# ICMP Rate Limiting

Generating ICMP error messages for every dropped packet would overwhelm the network. The Linux kernel rate-limits ICMP:

# Linux kernel ICMP rate limiting (sysctl)

net.ipv4.icmp_ratelimit = 1000          # ms between ICMPv4 error messages

net.ipv4.icmp_ratemask = 6168           # bitmask: which types are rate-limited

net.ipv6.icmp.ratelimit = 1000          # ms between ICMPv6 error messages

# Token bucket: allows bursts up to the bucket size,

# then limits to 1 message per ratelimit ms.

# Default: 1 error per second per destination

## Rate Limiting Implications

- Traceroute: If you send probes too fast, intermediate routers may rate-limit their ICMP Time Exceeded replies, showing * (no response) for some hops.

- PMTUD: Rate limiting can delay Packet Too Big responses, causing temporary PMTUD black holes under heavy load.

- Monitoring: ICMP-based monitoring (ping checks) may show false positives during rate limiting.

# ICMPv6 and Neighbor Discovery (NDP)

ICMPv6 types 133–137 form the Neighbor Discovery Protocol, which is essential for IPv6 operation:

| Type | Message | Function | IPv4 Equivalent |
| --- | --- | --- | --- |
| 133 | Router Solicitation | Host asks for router info | DHCP Discover (partial) |
| 134 | Router Advertisement | Router announces prefix, MTU, flags, DNS | DHCP Offer + gateway info |
| 135 | Neighbor Solicitation | Resolve IPv6 → MAC; DAD check | ARP Request |
| 136 | Neighbor Advertisement | Reply with MAC address | ARP Reply |
| 137 | Redirect | Better next-hop notification | ICMP Redirect |

## Neighbor Solicitation/Advertisement Detail

Neighbor Solicitation (Type 135):

+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+

| Type=135      |    Code=0     |          Checksum             |

+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+

|                         Reserved                              |

+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+

|                                                               |

+                    Target Address (128 bits)                  +

|                                                               |

+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+

| Options: Source Link-Layer Address (sender's MAC)             |

+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+

Sent to solicited-node multicast: ff02::1:ffXX:XXXX

(last 24 bits of target address)

# ICMP Firewall Rules

ICMP requires careful firewall handling. Blocking too much breaks the network; allowing too much enables reconnaissance.

## Essential ICMPv4 to Allow

# Minimum ICMPv4 ruleset (iptables)

iptables -A INPUT -p icmp --icmp-type destination-unreachable -j ACCEPT

iptables -A INPUT -p icmp --icmp-type time-exceeded -j ACCEPT

iptables -A INPUT -p icmp --icmp-type parameter-problem -j ACCEPT

iptables -A INPUT -p icmp --icmp-type echo-request -j ACCEPT  # allow ping

iptables -A INPUT -p icmp --icmp-type echo-reply -j ACCEPT

# CRITICAL: Allow fragmentation needed (PMTUD)

iptables -A INPUT -p icmp --icmp-type fragmentation-needed -j ACCEPT

## Essential ICMPv6 to Allow

# Minimum ICMPv6 ruleset (ip6tables)

# Error messages (MUST allow)

ip6tables -A INPUT -p icmpv6 --icmpv6-type destination-unreachable -j ACCEPT

ip6tables -A INPUT -p icmpv6 --icmpv6-type packet-too-big -j ACCEPT

ip6tables -A INPUT -p icmpv6 --icmpv6-type time-exceeded -j ACCEPT

ip6tables -A INPUT -p icmpv6 --icmpv6-type parameter-problem -j ACCEPT

# Ping

ip6tables -A INPUT -p icmpv6 --icmpv6-type echo-request -j ACCEPT

ip6tables -A INPUT -p icmpv6 --icmpv6-type echo-reply -j ACCEPT

# NDP (CRITICAL — IPv6 BREAKS without these)

ip6tables -A INPUT -p icmpv6 --icmpv6-type router-solicitation -j ACCEPT

ip6tables -A INPUT -p icmpv6 --icmpv6-type router-advertisement -j ACCEPT

ip6tables -A INPUT -p icmpv6 --icmpv6-type neighbour-solicitation -j ACCEPT

ip6tables -A INPUT -p icmpv6 --icmpv6-type neighbour-advertisement -j ACCEPT

# MLD (multicast)

ip6tables -A INPUT -p icmpv6 --icmpv6-type 130 -j ACCEPT

ip6tables -A INPUT -p icmpv6 --icmpv6-type 131 -j ACCEPT

ip6tables -A INPUT -p icmpv6 --icmpv6-type 132 -j ACCEPT

## nftables (Modern)

table inet filter {

chain input {

type filter hook input priority 0; policy drop;

ct state established,related accept

iif lo accept

# ICMPv4

ip protocol icmp icmp type {

destination-unreachable, time-exceeded,

parameter-problem, echo-request, echo-reply

} accept

# ICMPv6 (CRITICAL)

ip6 nexthdr icmpv6 icmpv6 type {

destination-unreachable, packet-too-big,

time-exceeded, parameter-problem,

echo-request, echo-reply,

nd-router-solicit, nd-router-advert,

nd-neighbor-solicit, nd-neighbor-advert,

mld-listener-query, mld-listener-report, mld-listener-done

} accept

}

}

# ICMP-Based Monitoring

## Smokeping / fping

# fping: parallel ping across multiple targets

fping -c 10 -q 8.8.8.8 1.1.1.1 9.9.9.9

# Output: loss%, min/avg/max RTT per target

# Smokeping: continuous latency/loss graphing

# Sends pings every 5 minutes, plots latency + loss

# Perfect for detecting intermittent packet loss

## Blackbox Exporter (Prometheus)

# prometheus.yml

scrape_configs:

- job_name: 'icmp'

metrics_path: /probe

params:

module: [icmp]

static_configs:

- targets:

- 8.8.8.8

- 1.1.1.1

relabel_configs:

- source_labels: [__address__]

target_label: __param_target

- target_label: __address__

replacement: blackbox-exporter:9115

Metrics: probe_icmp_duration_seconds, probe_success, probe_icmp_reply_hop_limit. Alert on sustained loss or latency exceeding SLA thresholds.

“The Pattern echoes in every error message.” — The Whispering Void