THE UNHEADED KINGDOM

ASSEMBLY LANGUAGE TUTORIAL SERIES

Part 3 of 4

Advanced Subnetting,
VLSM & IPv6
Addressing

Classful History, VLSM, Route Summarization, IPv6 Addressing Architecture, and Network Design

For Infrastructure Engineers

February 2026

bellis.tech

# Classful Addressing (Historical)

Before CIDR (1993), IP addresses were divided into fixed classes:

| Class | First Octet | Default Mask | Networks | Hosts/Network | Range |
| --- | --- | --- | --- | --- | --- |
| A | 1–126 | /8 (255.0.0.0) | 126 | 16,777,214 | 1.0.0.0 – 126.255.255.255 |
| B | 128–191 | /16 (255.255.0.0) | 16,384 | 65,534 | 128.0.0.0 – 191.255.255.255 |
| C | 192–223 | /24 (255.255.255.0) | 2,097,152 | 254 | 192.0.0.0 – 223.255.255.255 |
| D | 224–239 | N/A (multicast) | N/A | N/A | 224.0.0.0 – 239.255.255.255 |
| E | 240–255 | N/A (reserved) | N/A | N/A | 240.0.0.0 – 255.255.255.255 |

The problem: massive waste. An organization needing 300 hosts got a Class B with 65,534 addresses. CIDR (RFC 4632) replaced this with variable-length prefixes, enabling efficient allocation.

# Variable-Length Subnet Masking (VLSM)

VLSM allows different subnets within the same network to have different prefix lengths, matching subnet sizes to actual needs:

## Design Example: Office with Mixed Needs

Given: 10.1.0.0/24 (256 addresses)

Requirements:

Engineering:    100 hosts -> need /25 (126 usable)

Sales:           50 hosts -> need /26 (62 usable)

Management:      20 hosts -> need /27 (30 usable)

Server VLAN:     10 hosts -> need /28 (14 usable)

Point-to-point:   2 hosts -> need /30 (2 usable)

Allocation (largest to smallest):

10.1.0.0/25     Engineering   (10.1.0.1 - 10.1.0.126)

10.1.0.128/26   Sales         (10.1.0.129 - 10.1.0.190)

10.1.0.192/27   Management    (10.1.0.193 - 10.1.0.222)

10.1.0.224/28   Servers       (10.1.0.225 - 10.1.0.238)

10.1.0.240/30   P2P link      (10.1.0.241 - 10.1.0.242)

10.1.0.244/30   P2P link 2    (10.1.0.245 - 10.1.0.246)

10.1.0.248/29   Future use    (8 addresses spare)

Key rule: always allocate largest subnets first to prevent fragmentation of the address space.

# Route Summarization (Supernetting)

Summarization combines multiple contiguous subnets into a single routing table entry, reducing routing table size:

Four subnets to summarize:

10.1.0.0/24   = 10.1.00000000.0/24

10.1.1.0/24   = 10.1.00000001.0/24

10.1.2.0/24   = 10.1.00000010.0/24

10.1.3.0/24   = 10.1.00000011.0/24

^^^^^^

Common prefix: 10.1.000000

= 22 bits match

Summary route: 10.1.0.0/22 (covers all four /24s)

Verification:

10.1.0.0/22 range: 10.1.0.0 - 10.1.3.255 ✓

## Summarization Rules

- Power-of-2 alignment: Subnets must start at a boundary that is a multiple of the block size. 10.1.0.0/22 starts at 0 (0 mod 4 = 0). 10.1.4.0/22 starts at 4 (4 mod 4 = 0). But 10.1.1.0/22 is NOT valid because 1 mod 4 ≠ 0.

- Contiguous: All subnets in the summary must be contiguous (no gaps).

- No overlap: The summary must not accidentally include networks not intended.

# IPv6 Addressing Architecture

## Address Structure

Global Unicast Address (GUA): 2001:0db8:85a3:0001:0000:8a2e:0370:7334

|<-- Global Routing Prefix -->|<-SID->|<-- Interface ID (64 bits) -->|

|      48 bits (typical)      |16 bits|         64 bits              |

Global Routing Prefix: Assigned by ISP/RIR (identifies the organization)

Subnet ID (SID):       Organization's internal subnets (2^16 = 65,536)

Interface ID:          Identifies the host (2^64 per subnet!)

## IPv6 Prefix Lengths

| Prefix | Allocation Level | Subnets Available | Use |
| --- | --- | --- | --- |
| /32 | ISP allocation from RIR | 65,536 /48s | Large ISP |
| /48 | Standard site allocation | 65,536 /64s | Enterprise / campus |
| /56 | Residential / small biz | 256 /64s | Home network |
| /64 | Single subnet (standard) | 1 subnet | One VLAN / segment |
| /127 | Point-to-point (RFC 6164) | 2 addresses | Router-to-router links |
| /128 | Single host | 1 address | Loopback, host route |

## IPv6 Address Types by Prefix

| Prefix | Type | Scope |
| --- | --- | --- |
| 2000::/3 | Global Unicast (GUA) | Internet-routable |
| fc00::/7 (fd00::/8 in practice) | Unique Local (ULA) | Private (like RFC 1918 in IPv4) |
| fe80::/10 | Link-Local | Single link only (auto-generated) |
| ff00::/8 | Multicast | One-to-many delivery |
| ::1/128 | Loopback | Self (like 127.0.0.1) |
| ::/128 | Unspecified | No address yet (like 0.0.0.0) |
| ::ffff:0:0/96 | IPv4-mapped IPv6 | IPv4 address embedded in IPv6 format |
| 64:ff9b::/96 | NAT64 well-known prefix | IPv4 destination translated to IPv6 |

# Subnetting IPv6

Given: 2001:db8:abcd::/48 (standard site allocation)

Subnet ID field: bits 49-64 (16 bits = 65,536 subnets)

Example subnet plan:

2001:db8:abcd:0000::/64  -> Management

2001:db8:abcd:0001::/64  -> Engineering

2001:db8:abcd:0002::/64  -> Sales

2001:db8:abcd:0010::/64  -> Server VLAN 1

2001:db8:abcd:0011::/64  -> Server VLAN 2

2001:db8:abcd:0100::/64  -> Guest WiFi

2001:db8:abcd:0200::/64  -> IoT devices

2001:db8:abcd:ffff::/64  -> Out-of-band management

Each /64 subnet has 2^64 = 18,446,744,073,709,551,616 addresses.

You will never exhaust a /64. Ever.

# Special IPv4 Addresses

| Address/Range | Name | Purpose |
| --- | --- | --- |
| 0.0.0.0 | Default / unspecified | "This host on this network" or default route |
| 127.0.0.0/8 | Loopback | Traffic to self; never leaves the host |
| 169.254.0.0/16 | Link-Local (APIPA) | Auto-assigned when no DHCP response |
| 10.0.0.0/8 | Private (RFC 1918) | Large private networks |
| 172.16.0.0/12 | Private (RFC 1918) | Medium private networks |
| 192.168.0.0/16 | Private (RFC 1918) | Small private networks |
| 192.0.2.0/24 | Documentation (TEST-NET-1) | For examples and documentation only |
| 198.51.100.0/24 | Documentation (TEST-NET-2) | For examples and documentation only |
| 203.0.113.0/24 | Documentation (TEST-NET-3) | For examples and documentation only |
| 100.64.0.0/10 | Shared (CGN / RFC 6598) | Carrier-grade NAT space |
| 255.255.255.255 | Limited broadcast | Broadcast to local network segment |

# Practical Tools

# ipcalc: instant subnet info

ipcalc 192.168.1.0/26

Address:   192.168.1.0

Netmask:   255.255.255.192 = 26

Wildcard:  0.0.0.63

Network:   192.168.1.0/26

HostMin:   192.168.1.1

HostMax:   192.168.1.62

Broadcast: 192.168.1.63

Hosts/Net: 62

# sipcalc: IPv6-aware

sipcalc 2001:db8::/48

# Python quick math

python3 -c "import ipaddress; n=ipaddress.ip_network('10.0.0.0/22')

for s in n.subnets(new_prefix=24): print(s)"

# Linux: show addresses with prefix

ip -4 addr show        # IPv4 with CIDR

ip -6 addr show        # IPv6 with prefix length

# Linux: add address

ip addr add 10.1.0.1/24 dev eth0

ip addr add 2001:db8::1/64 dev eth0

# Linux: show routes

ip route show          # IPv4 routing table

ip -6 route show       # IPv6 routing table

“The Pattern is written in ones and zeros.” — The Whispering Void