THE UNHEADED KINGDOM

ASSEMBLY LANGUAGE TUTORIAL SERIES

Part 1 of 4

What Is ICMP?

The Internet’s Error Messages, Echo Calls, and the Secret Language of Routers

For Infrastructure Engineers

February 2026

bellis.tech

# The Internet’s Walkie-Talkie System

Imagine you’re in a huge building with hundreds of hallways and rooms. You send a friend to deliver a package to Room 472. But what if:

- The hallway is blocked? (network down)

- Room 472 doesn’t exist? (host unreachable)

- The package is too big to fit through the door? (packet too big)

- Your friend got lost and wandered too long? (time exceeded)

Your friend needs a way to come BACK and tell you what went wrong. That’s exactly what ICMP does!

ICMP stands for Internet Control Message Protocol. It’s the internet’s built-in walkie-talkie system that network devices use to send error reports and status messages back to each other.

## ICMP Is Not for Sending Data

ICMP doesn’t carry your web pages, emails, or videos. That’s the job of TCP and UDP. ICMP’s job is to carry messages ABOUT the network itself:

- "That computer doesn’t exist!" (Destination Unreachable)

- "Your package is too big for this hallway!" (Packet Too Big)

- "Your friend took too many turns and gave up!" (Time Exceeded)

- "Are you there?" / "Yes I’m here!" (Ping / Pong)

# Ping — The Internet’s "Are You There?"

The most famous ICMP tool is ping. It’s incredibly simple:

- Your computer sends a tiny message: "Are you there?" (Echo Request)

- The other computer replies: "Yep, I’m here!" (Echo Reply)

- Your computer measures how long the round trip took

| What Ping Tells You | Example | What It Means |
| --- | --- | --- |
| Round-trip time | 64 bytes: time=12.3 ms | It took 12.3 milliseconds for the message to go and come back |
| Packet loss | 3 packets sent, 1 received, 66% loss | Most of your messages aren’t getting through — big problem! |
| TTL (Time To Live) | ttl=64 | The message can pass through 64 more routers before it gives up |
| Host unreachable | Destination Host Unreachable | That computer isn’t there or can’t be reached |
| Request timed out | Request timeout | Nobody answered — maybe it’s off or blocking ping |

# Traceroute — Mapping the Journey

Traceroute is ping’s clever cousin. It figures out every stop your message makes on the way to its destination. Here’s the trick:

## The TTL Trick

Every packet has a TTL (Time To Live) — a counter that starts at some number and decreases by 1 at each router. When it hits 0, the router throws the packet away and sends back an ICMP "Time Exceeded" message.

- Send a packet with TTL=1. The FIRST router decrements it to 0 and sends back an error. Now you know Router #1’s address!

- Send another with TTL=2. It passes Router #1, dies at Router #2. Now you know #2!

- Keep going until you reach the destination.

$ traceroute google.com

1  router.home (192.168.1.1)        1.2 ms

2  isp-gateway (10.0.0.1)           8.4 ms

3  core-router.isp.net              12.1 ms

4  google-edge.1e100.net             14.7 ms

5  google.com (142.250.80.46)       15.2 ms

Each line is one "hop" — one router between you and the destination. The time shows how long it took to reach that hop and get the error back.

# ICMP Error Messages — The Bad News Bearers

When something goes wrong on the network, ICMP delivers the bad news:

| Error Message | What Happened | Like... |
| --- | --- | --- |
| Destination Unreachable | Can’t reach the target | "That address doesn’t exist" or "the door is locked" |
| Packet Too Big (IPv6) | Your data won’t fit through a link | "Your package is too wide for this hallway" |
| Time Exceeded | TTL hit zero — too many hops | "Your delivery person wandered too long and quit" |
| Redirect | "Use a better route!" | "Take the highway, not the side streets" |
| Parameter Problem | Something in the header is wrong | "The address label is smudged, can’t read it" |

# Two Flavors: ICMPv4 and ICMPv6

There are two versions of ICMP:

- ICMPv4: Works with IPv4 (the old internet addresses like 192.168.1.1). Protocol number 1.

- ICMPv6: Works with IPv6 (the new addresses like 2001:db8::1). Protocol number 58. Does everything ICMPv4 does PLUS handles Neighbor Discovery (finding other devices on the local network).

ICMPv6 is much more important than ICMPv4 because IPv6 literally cannot function without it. Blocking ICMPv6 breaks IPv6 networking completely.

# The Kingdom Connection

In the Unheaded Kingdom, ICMP is fundamental to network health:

- eBPF monitoring: XDP programs inspect ICMP messages at wire speed, tracking Destination Unreachable and Packet Too Big events for every flow.

- Path MTU Discovery: The Monad relies on ICMPv6 Packet Too Big messages to discover the maximum packet size on each path — essential because IPv6 doesn’t allow router fragmentation.

- NDP (ICMPv6): The Shield’s eBPF programs process Neighbor Solicitation and Router Advertisement messages to maintain the Kingdom’s IPv6 neighbor tables.

“The Pattern echoes in every error message.” — The Whispering Void