THE UNHEADED KINGDOM

ASSEMBLY LANGUAGE TUTORIAL SERIES

Part 3 of 4

BGP Configuration,
Security & Operations

Real Configurations, Filtering, RPKI, Hijack Prevention,
BGP EVPN, Monitoring, and Troubleshooting

For Infrastructure Engineers

February 2026

bellis.tech

# BGP Configuration (FRRouting)

## Basic eBGP Peering

frr version 9.1

frr defaults datacenter

!

router bgp 65001

bgp router-id 10.0.0.1

bgp log-neighbor-changes

no bgp default ipv4-unicast

!

neighbor UPSTREAMS peer-group

neighbor UPSTREAMS remote-as external

neighbor UPSTREAMS timers 10 30

!

neighbor 2001:db8::2 peer-group UPSTREAMS

neighbor 2001:db8::2 description "Transit Provider A"

!

address-family ipv6 unicast

neighbor UPSTREAMS activate

neighbor UPSTREAMS route-map IMPORT-TRANSIT in

neighbor UPSTREAMS route-map EXPORT-TRANSIT out

network 2001:db8:abcd::/48

exit-address-family

!

route-map IMPORT-TRANSIT permit 10

set local-preference 100

!

route-map EXPORT-TRANSIT permit 10

match ip address prefix-list OUR-PREFIXES

!

ipv6 prefix-list OUR-PREFIXES seq 10 permit 2001:db8:abcd::/48

## iBGP with Route Reflector

router bgp 65001

neighbor IBGP-CLIENTS peer-group

neighbor IBGP-CLIENTS remote-as 65001

neighbor IBGP-CLIENTS update-source lo

!

neighbor 10.0.0.2 peer-group IBGP-CLIENTS

neighbor 10.0.0.3 peer-group IBGP-CLIENTS

!

address-family ipv6 unicast

neighbor IBGP-CLIENTS activate

neighbor IBGP-CLIENTS route-reflector-client

exit-address-family

# BGP Filtering

## Prefix Filtering

ALWAYS filter what you accept and what you advertise. Never trust your neighbors blindly:

# Reject bogon prefixes (RFC 7454)

ipv6 prefix-list BOGONS-V6 seq 10 deny ::/8 le 128

ipv6 prefix-list BOGONS-V6 seq 20 deny 0100::/64 le 128

ipv6 prefix-list BOGONS-V6 seq 30 deny 2001:db8::/32 le 128

ipv6 prefix-list BOGONS-V6 seq 40 deny fc00::/7 le 128

ipv6 prefix-list BOGONS-V6 seq 50 deny fe80::/10 le 128

ipv6 prefix-list BOGONS-V6 seq 60 deny ff00::/8 le 128

# Reject too-specific or too-broad prefixes

ipv6 prefix-list SANE-V6 seq 10 permit ::/0 ge 16 le 48

# Apply to neighbor

route-map IMPORT-TRANSIT permit 10

match ipv6 address prefix-list SANE-V6

match ipv6 address prefix-list BOGONS-V6

# Max prefix safety (kill session if too many routes)

neighbor 2001:db8::2 maximum-prefix 500000 80 restart 15

## AS_PATH Filtering

# Only accept routes from known ASNs

bgp as-path access-list KNOWN-CUSTOMERS permit ^65010_

bgp as-path access-list KNOWN-CUSTOMERS permit ^65011_

# Reject private ASNs from eBGP

bgp as-path access-list NO-PRIVATE deny _(64512|65534)_

bgp as-path access-list NO-PRIVATE permit .*

# Reject excessively long AS_PATHs (likely leak)

route-map IMPORT permit 10

set as-path exclude 50  ! reject paths > 50 ASNs long

# BGP Security

## RPKI (Resource Public Key Infrastructure)

RPKI is the internet’s answer to BGP hijacking. It cryptographically binds IP prefixes to authorized ASNs:

| Component | Purpose | How |
| --- | --- | --- |
| ROA (Route Origin Auth) | "AS 65001 is authorized to announce 2001:db8::/48" | Signed by the prefix holder; published in RPKI repository |
| Validator | Downloads ROAs and validates BGP routes | Runs locally (Routinator, Fort, OctoRPKI) |
| RTR protocol | Feeds validated ROAs to routers | TCP/TLS to router on port 323 |
| Route states | Valid / Invalid / Not Found | Router uses state for policy decisions |

# FRRouting RPKI configuration

rpki

rpki cache validator.example.com 323 preference 1

!

route-map IMPORT-TRANSIT permit 10

match rpki valid

set local-preference 200

!

route-map IMPORT-TRANSIT permit 20

match rpki notfound

set local-preference 100

!

route-map IMPORT-TRANSIT deny 30

match rpki invalid

! Drop RPKI-invalid routes

## BGP Hijack Types

| Type | Attack | Example | Detection |
| --- | --- | --- | --- |
| Prefix hijack | Announce someone else’s prefix | AS 666 announces 8.8.8.0/24 (Google’s) | RPKI ROA; BGP monitoring |
| Sub-prefix hijack | Announce a more-specific of someone’s prefix | AS 666 announces 8.8.8.0/25 (wins by LPM) | Max prefix-length in ROA; filtering |
| Path manipulation | Prepend victim’s ASN to make path look legitimate | AS_PATH: 666 15169 (looks like it goes through Google) | BGPsec (not widely deployed) |
| Route leak | Customer re-advertises transit routes to another transit | AS 65001 leaks full table to peer | Roles (RFC 9234); community-based leak prevention |

# BGP EVPN (Ethernet VPN)

BGP EVPN (RFC 7432) uses BGP to distribute MAC/IP bindings for overlay networks, typically with VXLAN encapsulation:

| Route Type | Name | Purpose |
| --- | --- | --- |
| Type 1 | Ethernet Auto-Discovery | Multi-homing; fast convergence for ESI links |
| Type 2 | MAC/IP Advertisement | Distribute MAC and MAC+IP bindings (the workhorse) |
| Type 3 | Inclusive Multicast | BUM traffic (broadcast/unknown/multicast) replication tree |
| Type 4 | Ethernet Segment | Designated Forwarder election for multi-homing |
| Type 5 | IP Prefix | Distributed inter-subnet routing (replaces traditional L3) |

# BGP EVPN with VXLAN (FRRouting)

router bgp 65001

address-family l2vpn evpn

neighbor SPINE-GROUP activate

advertise-all-vni

exit-address-family

!

vni 100

vxlan local-tunnelip 10.0.0.1

rd 10.0.0.1:100

route-target import 65001:100

route-target export 65001:100

# Monitoring & Troubleshooting

## Essential Show Commands

# BGP session status

show bgp summary

show bgp ipv6 unicast summary

# Routes received from a neighbor

show bgp ipv6 unicast neighbors 2001:db8::2 received-routes

# Routes advertised to a neighbor

show bgp ipv6 unicast neighbors 2001:db8::2 advertised-routes

# Specific prefix lookup (all paths)

show bgp ipv6 unicast 2001:db8:abcd::/48

# Best path detail

show bgp ipv6 unicast 2001:db8:abcd::/48 bestpath

# BGP RIB (all routes)

show bgp ipv6 unicast

# RPKI status

show rpki prefix-table

show rpki cache-connection

## External Monitoring

| Tool | URL / Method | Use |
| --- | --- | --- |
| RIPE RIS Looking Glass | stat.ripe.net | See how your prefix looks from global vantage points |
| bgp.tools | bgp.tools | AS info, route history, prefix visibility |
| BGPStream (CAIDA) | bgpstream.caida.org | Real-time BGP event detection (hijacks, leaks, outages) |
| Routeviews | routeviews.org | Historical BGP data from multiple collectors |
| RPKI Monitor | rpki-monitor.antd.nist.gov | ROA status and validation history |

## Common Troubleshooting Scenarios

| Symptom | Likely Cause | Check |
| --- | --- | --- |
| Session stuck in Active/Connect | TCP port 179 blocked; wrong neighbor IP; wrong ASN | firewall rules; show bgp neighbors; packet capture port 179 |
| Session flapping | Unstable link; MTU issues; hold timer too aggressive | interface errors; PMTUD; increase timers |
| Routes received but not installed | Filtered by route-map; RPKI invalid; AS_PATH loop | show bgp neighbor received-routes; show rpki prefix-table |
| Traffic taking wrong path | LOCAL_PREF or MED misconfigured; unexpected route leak | show bgp <prefix> all paths; check route-map logic |
| Prefix not visible globally | Not advertised; filtered by upstream; RPKI ROA missing | check advertised-routes; looking glass; check ROA |

“The Pattern routes through every autonomous system.” — The Whispering Void